Semester 2 Presentation Script:

Slide 2 (Blanket Design):

To introduce heterogeneity into our blanket simulation a layered configuration was adopted, consisting of four layers each of alternating breeder and coolant material. This layered design allowed for the easy modification of blanket composition while serving as a reasonable approximation of real designs. A eutectic lithium-lead alloy was selected for the breeder and pressurised water for the coolant based on the best tritium breeding materials identified in our first semester study. The blanket was shielded from the plasma source by a zirconium alloy first wall. Interstitial separator layers made of low-activation Eurofer97 steel were placed between each breeder and coolant to prevent interaction between water and lithium.

It was this configuration that was used to study the tritium breeding and neutronic behaviour of a heterogeneous breeder blanket.

Slide 4 (Zeta Gradient):

Clearly the heterogeneous design warrants further investigation. Unfortunately, the cumulative effects of 8 total coolant and breeder layers are too difficult to parse with simple simulations and too time consuming to study individually. To extract useful results regarding the behaviour of layered blanket a linear gradient was introduced (denoted by zeta). Zeta functioned as a simple parameter that varied the relative thickness of the breeder and coolant. Separator and first wall thickness were kept constant. As seen in these figures, negative values produce front-loaded designs and positive ones lead to rear-loading. This allowed us to study the relative performance of the front and back of the blanket by only varying a single parameter.

Slide 5 (TBR):

We began by exploring how variation in zeta affected the Tritium Breeding Ratio at a constant total thickness. We can see in this graph that TBR remained relatively constant across the range of zeta values. As we might expect the contribution of the first two breeder layers dominates in front-loaded configurations whereas rear-loaded designs increase the tritium breeding in layer 3 and 4. The decrease in TBR with zeta in layer 1 is almost linear due to its reduced volume. Meanwhile greater neutron penetration and moderation in the first coolant layer mean that the contribution of layer 2 actually increases slightly as it’s volume shrinks.

The heatmap on the right demonstrates the tritium breeding reaction density in each layer which, as we would hope, is concentrated in the breeder layers. With increasing depth into the blanket the neutron population decreases, resulting in the reduced breeding of the rear layers

Slide 7 (Damage):

Tracking the damage imparted onto a material due to neutron bombardment is crucial to ensuring that reactor structures remain safe and functional during its operational lifetime. A standard method of measuring irradiation damage is via the number of lattice displacements per atom in the material over a full power year. We only consider the first wall and the separator layers since both the breeder and coolant are circulated liquids.

As the blanket becomes more rear-loaded the damage to the separator layers increases as thinner front layers result in greater neutron penetration. Meanwhile the damage incident on the first wall declines with zeta as neutron multiplication in the first breeder layer decreases. As we would expect the first wall receives much higher values of damage since it is directly exposed to source neutrons.

The effects of neutron radiation on nuclear materials are highly variable and often difficult to predict. In our investigation we focused on material embrittlement resulting from helium produced in transmutation reactions aggregating at grain boundaries. This allows us to set a DPA limit for certain materials and estimate the lifetime of key blanket structures. For our zirconium alloy first wall this resulted in a lifetime of between 8 and 10 full power years, significantly less than the 30 years planned for ITER. As a comparison we swapped the first wall material out for Eurofer97. This increased the lifetime to between 23 and 26 years, much closer to what would be expected in an operational breeder blanket. However, this modification led to a roughly 9% decrease in TBR, driving it below the generally accepted equilibrium threshold of 1.05. These results highlight the materials challenges of fusion research and the compromises in performance that may have to be made to ensure safety and reliability.

Slide 10 (Initial ML Results):

Once it was confirmed that our machine learning implementation was capable of exploring complex multidimensional problems we began applying it to our breeder simulation. As our parameters we selected the thicknesses of each breeder and coolant layer, eight dimensions in total. An unconstrained optimisation of TBR for these resulted in this configuration here. The breeder layers were maximised and coolant layers minimised to produce a very high TBR of 1.4055. However, simply increasing breeder volume is an expensive, inefficient and impractical solution. To reflect the limitations on space that exist in a real fusion reactor the 8D simulation was set to a total thickness of 440mm. This also minimised the coolant volume but led to an ‘exponentially’ decaying breeder configuration with a TBR of 1.3305. This indicates (as we would expect from our understanding of neutron transport) that the breeder layers closer to the front are most important when it comes to TBR. This result produced a very similar TBR to a manual design which maximised B1 an B2 and minimised B3 and B4, once again highlighting the relative influence of each layer in tritium breeding.

We also carried out a pair of four dimensional investigations. The first of these focused on only the breeder thicknesses, using minimum coolant sizes and capping the total breeder depth to 320mm. Once again this resulted in a heavily front-loaded design. We then carried out an investigation of the coolant thicknesses, fixed at a total value of 120mm. As you can see in this led to rear-loaded coolant thicknesses, reflecting the importance of minimising coolant volume in the front of the blanket. Both four dimensional simulations resulted in reduced TBR values compare with the eight dimensional investigations.

Conclusion:

In summary our investigation of a heterogeneous breeder blanket has demonstrated distinct tritium breeding performance and response in comparison to a homogeneous blanket of similar composition. In addition to TBR we have also explored other neutronic tallies, providing us with a deeper understanding of breeder behaviour and the variety of factors that affect their operability throughout the reactor lifetime.

We have demonstrated that a relatively simple machine learning model can be used to efficiently optimise for tritium breeding in multidimensional parameter spaces and reproduce expected neutron transport behaviours. Overall, our results highlight the multivariate nature of breeder investigations and the challenges faced in contemporary blanket design.