MPhys Literature Review

Notes for literature review.

A literature review should establish the state of the field under consideration and provide the necessary context and background for the report. It establishes the scope of the research and ensures that projects are not simply repeating a previous experiment. It is NOT simply a list of relevant references.

Important refences are ones that cover experiments very similar to our or tell us about the technological state of play.

Key Points in Lit Review:

- The roles and requirements of breeder blankets in MCF fusion, primarily tritium production but also neutron reproduction, shielding and energy capture.

- Evaluation of the state of breeder blanket technology and research. Difference between solid and liquid breeders, which designs are being favoured and where are they being implemented/explored.

- How these papers influence are research, especially for projects very similar to ours. Where do they overlap and where do they differ? Can we expand on the subject or explore different directions? Projects that also use OpenMC are particularly useful.

Blanket & Desings:

As outlined above, deuterium-tritium fusion has long been considered to be the best candidate for a functional, energy producing reactor due to its large Q value and the production of 14MeV neutrons (Heckrotte and Hiskes, 1971). Unfortunately, while deuterium is very abundant and easy to obtain tritium is not found in nature due to its short half-life. Thus, tritium must be produced. Most of the world’s current tritium supply is generated in heavy water moderated fission reactors when neutrons interact with the deuterium in the water. However, as is detailed in Kovari et al. (2018) global tritium stockpiles are facing serious shortages even without demand from fusion reactors. This has incentivised the design of an in-situ method of tritium production known as a breeder blanket. Initially envisaged as a method of creating fissile materials for nuclear reactors (Moir, 1982) (Lee, 1983), a breeder blanket is a structure consisting of an element that produces desirable isotopes upon interacting with fusion neutrons. Thus, as shown by equation [x], a lithium-6 blanket structure could produce the tritium required to fuel a full scale fusion reactor. A significant body of research has been composed on lithium-based breeder blankets in the last decade in anticipation of a series of test reactors planned for the 2040’s and 50’s. Strategy documents are already outlining conceptual designs for blankets on ITER’s prospective DEMO reactors (Federici et al., 2019), the Chinese Fusion Engineering Test Reactor (CFETR) facility currently under construction (Liu et al., 2022) and the Spherical Tokamak for Energy Production (STEP) planned for the UK (Lord et al., 2024).

Due to the neutron economy of d-t fusion and the neutron-lithium interaction, any losses within the system would inevitably lead to a tritium deficit. To prevent this, modern blanket designs have moved beyond pure lithium structures. By including lead or beryllium isotopes spare neutrons can be generated, ensuring that the tritium breeding ratio (TBR) of the system remains is above equilibrium. TBR is a key indicator of blanket performance and can be used to effectively compare a variety of blanket compositions as demonstrated in (Hernández and Pereslavtsev, 2018). Breeder blankets can be divided into two main categories, solid breeders based on ceramic lithium oxides and liquid breeders consisting of molten lithium compounds. The key differences between these solid and liquid breeders are in terms of thermal properties and tritium extraction (Konishi et al., 2017), qualities that have limited impact on our project but are of considerable importance to the design of practical breeder blankets.

Solid breeder designs have been proposed for several breeder projects, most notably at the CFETR where a Water Cooled Pebble Bed (WCPB) design utilising a mixture of lithium metatitanate (Li2TiO3) and beryllide (Be12Ti) pellets has seen particular interest (Liu et al., 2022) (Lei et al., 2020). Alternatives using lithium orthosilicate (Li4SiO4) ceramics are also being explored as candidates for test modules including ITER’s DEMO program (Chen et al., 2015) (Wang et al., 2019).

In contrast to the more complex structures of solid breeders, liquid breeders allow for a much simpler design, providing breeding and cooling all at once and overcoming the packing constraints of pebble beds. The simplest liquid blanket designs involve a eutectic lithium-lead alloy (Tas et al., 1988) (Wu, 2011), typically chosen to be Li17Pb83. A eutectic mixture has a melting point lower than its constituent materials, a key property in liquid systems that require proper circulation (Mas de les Valls et al., 2008). As outlined in Fernández-Berceruelo et al. (2024) this is the basis of the Dual Cooled Lithium-Lead system currently being explored as an option for the DEMO reactor project. Systems involving more chemically complex molten salts are seeing significant interest as a breeder for the Affordable Robust Compact (ARC) reactor developed by the Massachusetts Institute of Technology (MIT). The ARC project is dedicated to utilising an FLiBe (2LiF – BeF2) blanket, combining breeder (lithium), multiplier (beryllium) and neutron absorber (fluorine) in one system with the goal of a reducing reactor size and increasing efficiency (Bocci et al., 2020) (Sorbom et al., 2015).

While many of these blanket designs have existed for decades (Shatalov et al., 1991) that is not to say that the field is bereft of innovation. Recent attempts have been made to devise a system that combines the strengths of solid and liquid breeders. As part of the STEP programme an encapsulated blanket design is being explored, consisting of a bed of solid pellets containing liquid FLiBe (Fradera et al., 2021). Recent material developments have also opened up research avenues that were initially seen as dead ends, expanding the range of viable blanket materials to molten chlorine salts (Bohm and Lindley, 2023) and demonstrating how even changes to the encasing material can have a marked impact on TBR (King et al., 2022).

Accurately describing neutron transport and interactions is key to evaluating the tritium breeding performance of fusion blanket designs. The probabilistic nature of neutron behaviour lends itself to modelling via a Monte Carlo simulation. This is clear to see in Wu (2011) and in Zheng and Todd (2015) where Monte Carlo codes are regularly used to model the neutronics of breeder designs planned for full scale international ventures. This project uses OpenMC, a Monte Carlo code designed specifically for neutron transport in CAD constructed environments. Bae, Peterson and Shimwell (2022) show that OpenMC demonstrates very close agreement with other neutron transport codes when modelling neutron flux spectra, power distribution and TBR. Our choice of programme is further justified by the examples of its use in studying the TBR of very complex blanket structures (Fradera et al., 2021) (Segantin, Testoni and Zucchetti, 2020).

Due to recent advances in the field and with construction of test reactors on the horizon much of the research associated with breeder blanket focusses on a small handful of materials and designs to ease implementation. Our goal is to study the neutronics of a wide variety of blanket materials. However, that is not to say that our project is unprecedented. The systematic modelling of TBR for a different blanket and properties has been explored before such as in Hernández and Pereslavtsev (2018). This study serves as a useful source of breeder materials to simulate and a useful point of reference as the project progresses. It also provides us with an opportunity to expand on our project in ways that the paper did not, namely exploring the behaviour of the TBR with varying enrichment and blanket thickness. A (currently) unpublished paper by Goel, Aslam and Dua (2023) also functions as a useful comparison to our project, in particular for its use of OpenMC and the Paramak Python module to model the TBR. Once again it contains useful details while providing us with interesting avenues for further study.

When evaluating our results it is not altogether possible to find a definitive TBR for a certain material at set conditions. This is because no currently operating fusion test reactors have built in breeder blankets and simulation results are dependent on the model geometry used. However, there are some small scale experiments that aim to study the functionality of practical blanket modules. Delaporte-Mathurin et al. (2025) explores the TBR for a 1000ml molten salt module and the unpredictability of tritium behaviour and Hirose et al. (2024) describes the construction of a mockup WCPB module. These studies do not play a direct role in the progress of our project however they emphasize the present difficulty of performing full scale breeder experiments and explain the computational nature of our project and the majority of blanket research.

Key Points:

It’s up to you whether you use all these points or what order you want to use them in. The text above is a draft of a lit review. If you have any questions or if you want to know anything about the references let me know.

- Why we use D-T fusion (Heckrotte and Hiskes, 1971)

- The issues that come from using tritium, in particular supply (Kovari et al., 2018) (Pearson, Antoniazzi and Nuttall, 2018)

- History of fusion reactors, initially used for fission isotope production (Moir, 1982) (Lee, 1983)

- Solid Breeders

  - Li2TiO3  (Liu et al., 2022) (Lei et al., 2020) (CFETR mainly)

  - Li4SiO4 (Chen et al., 2015) (Wang et al., 2019) (CFETR and DEMO)

- Liquid Breeders

  - eutectic lithium-lead alloy (Tas et al., 1988) (Wu, 2011) and Fernández-Berceruelo et al. (2024) in DEMO, more on eutectic materials in (Mas de les Valls et al., 2008)

  - FLiBe (Bocci et al., 2020) (Sorbom et al., 2015) mainly used in ARC reactor. ARC reactor people really like molten salts.

- Encapsulated STEP Blanket (Fradera et al., 2021)

- Monte Carlo codes used in Wu (2011) and in Zheng and Todd (2015) among others. OpenMC in particular used in (Fradera et al., 2021) (Segantin, Testoni and Zucchetti, 2020) and some more.

- Studies similar to ours Hernández and Pereslavtsev (2018) and Goel, Aslam and Dua (2023)